STEM MECHANICAL LAB/ MAKERSPACE INSPECTION

Location: Date: Auditor(s):

| Machine Safety |  |  |  | YES | NO | N/A |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Rotating parts on machines are adequately guarded. (Collars, couplings, cams, clutches, flywheels, shaft ends, spindles, meshing gears, and horizontal or vertical shafting) |  |  |  |  |  |
| 2 | In-running nip points are adequately guarded. (intermeshing gears, rolling mills, belt and its pulley, a chain and a sprocket, and a rack and pinion, rotating and fixed parts which create a shearing, crushing, or abrading action) |  |  |  |  |  |
| 3 | Reciprocating motions are not creating struck by or caught between hazards |  |  |  |  |  |
| 4 | Point of Operation hazards are adequately guarded. (cutters, shears, power press, milling machine, saws, jointers, forming rolls, portable power tools) |  |  |  |  |  |
| 5 | Fan blades are adequately guarded (7 feet or opening less than ½ inch) |  |  |  |  |  |
| 6 | Fixed machinery is anchored to prevent walking or moving |  |  |  |  |  |
| 7 | Eye and face protection is being used when exposed to flying particles |  |  |  |  |  |
| 8 | The machine is locked out when service or maintenance is being done |  |  |  |  |  |
| 9 | Machine safe zone demarcated |  |  |  |  |  |
| Chemical Storage |  |  |  |  |  |  |
| 1 | There is a Chemical inventory list for the shop |  |  |  |  |  |
| 2 | Safety Data Sheets (SDS) are readily available for all hazardous materials |  |  |  |  |  |
| 3 | All hazardous substances are properly labeled and stored |  |  |  |  |  |
| 4 | Satellite accumulation areas for wastes are properly maintained |  |  |  |  |  |
| 5 | Universal waste is labeled and stored properly (used florescent bulbls, batteries) |  |  |  |  |  |
| 6 | Are the emergency spill kits available |  |  |  |  |  |
| Compressed Air |  |  |  |  |  |  |
| 1 | Air used for cleaning is regulated to 30 psi or less |  |  |  |  |  |
| 2 | Pressure relieving tips are on all air hoses |  |  |  |  |  |
| 3 | Hearing protection is worn while using compressed air |  |  |  |  |  |
| Compressed Gas Cylinders |  |  |  |  |  |  |
| 1 | Gas cylinders properly secured |  |  |  |  |  |
| 2 | Cylinder capped when not in use |  |  |  |  |  |
| 3 | Fuel gases segregated from oxygen (>20 feet) |  |  |  |  |  |
| 4 | Pressure relieved from valves on cylinders not in use |  |  |  |  |  |

| Fire Safety |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Flammable liquids are stored in approved flammable cabinets |  |  |  |  |
| 2 | Flammable liquid cabinet is NOT located near ignition sources or exits |  |  |  |  |
| 3 | Exits properly marked if applicable and aisles are unobstructed |  |  |  |  |
| 4 | Fire extinguisher available and employee has been trained |  |  |  |  |
| Electrical Safety |  |  |  |  |  |
| 1 | Damaged, defective equipment has been removed from service (missing ground prongs, cut or pinched cords) |  |  |  |  |
| 2 | Hand-held power tools are either grounded or marked as 'double insulated' |  |  |  |  |
| 3 | GFCIs are used in wet or damp locations |  |  |  |  |
| 4 | Circuit breaker panels are unobstructed |  |  |  |  |
| 5 | Extension cords are rated for heavy duty |  |  |  |  |
| 6 | Extension cords are in good condition |  |  |  |  |
| 7 | Extension cords are protected from damange (not run through doors, windows, on floors where they can be ran over with heavy equipment) |  |  |  |  |
| 8 | The shop is free from recognized electrical hazards (missing knockouts, missing circuit breakers, missing/broken/damaged covers, exposed live electrical componenets) |  |  |  |  |
| Housekeeping |  |  |  |  |  |
| 1 | The shop floor is free from slip, trip and fall hazards (water, oil, debrix etc.) |  |  |  |  |
| 2 | Shop materials are stored in a safe manner (including scrap) |  |  |  |  |
| 3 | Shop tools are safely stored and not left on machines |  |  |  |  |
| 4 | Oily rags are stored in appropriate metal containers |  |  |  |  |
| General Safety |  |  |  |  |  |
| 1 | Employees/students are not wearing loose clothing or jewelry |  |  |  |  |
| 2 | Employee/student completed the machine shop safety training |  |  |  |  |
| 3 | Long loose hair must be contained under a cap, scarf or other method |  |  |  |  |
| 4 | Required signs are posted |  |  |  |  |
| 5 | Protective eyewear is worn when working on or near machines |  |  |  |  |
| 6 | Manufacturer's manual or other reference manuals are available |  |  |  |  |
| 7 | Eyewash station is available and properly maintained |  |  |  |  |
| 8 | IH testing completed and IH procedures followed |  |  |  |  |
| 9 | Regulatory procedures followed |  |  |  |  |
| 10 | First aid kit available/sealed |  |  |  |  |
|  | Notes: |  |  |  |  |